Radiated Emission

Description: MIL-461F RE102 10 kHz to 30 MHz (3)

Time of measurement: 06 December 2016 10:51:59

Frequency range: 10 kHz to 30 MHz

Detected Peaks:

| Selected | Selected_RAW | Peak Number | Peak Number_RAW | Frequency | Frequency_RAW | Peak | Peak_RAW | Peak Limit | Peak Limit_RAW | Peak Difference | Peak Difference_RAW | Peak Correction | Peak Correction_RAW | Peak Status | Peak Status_RAW | Status | Status_RAW | Angle | Angle_RAW | Height | Height_RAW | Polarization | Polarization_RAW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| True | True | 1 | 1 | 9.956 MHz | 9.95649038461538 | 42 dBµV/m | 42.0377949781106 | 24 dBµV/m | 24 | 18.0 dB | 18.0377949781106 | -5.2 dB | -5.20220502188937 | Fail | Fail | Fail | Fail | 0 Degree | 0 | 1 cm | 0.01 | Vertical | Vertical |
| True | True | 2 | 2 | 19.954 MHz | 19.9543269230769 | 40.2 dBµV/m | 40.2002275452311 | 24 dBµV/m | 24 | 16.2 dB | 16.2002275452311 | -4.5 dB | -4.48977245476892 | Fail | Fail | Fail | Fail | 0 Degree | 0 | 1 cm | 0.01 | Vertical | Vertical |
| True | True | 3 | 3 | 29.952 MHz | 29.9521634615385 | 44.7 dBµV/m | 44.7331302923862 | 24 dBµV/m | 24 | 20.7 dB | 20.7331302923862 | -3.9 dB | -3.90686970761383 | Fail | Fail | Fail | Fail | 0 Degree | 0 | 1 cm | 0.01 | Vertical | Vertical |

||SELECT NR,[PEAK NR],FREQUENCY,PEAK,[PEAK LIMIT],[PEAK DIFFERENCE]||

Settings Analyzer:

| Ref. Level: | 97 dBµV |  | RBW: | 1 kHz |
| --- | --- | --- | --- | --- |
| Attenuator: | 10.0 dB |  | VBW: | 1 kHz |
| Preamp: | 0 dB |  | Sweeptime: | 2 s |
| Antenna: | Vertical at 1 m |  |  |  |

| Note: | Re-test |
| --- | --- |

Environment:

| Mode of operation: | ¦¦OPERATION MODE¦¦ |
| --- | --- |
| Test Engineer: | Henk Heutink |
| Temperature: | ¦¦TEMPERATURE¦¦ |
| Humidity: | ¦¦HUMIDITY¦¦ |